March 10, 2021

Communications Security, Reliability, and Interoperability Council VII

Report on recommendations to resolve Duplicate National Weather service alerts

Working Group 1:  Alert Originator Standard Operating Procedures-Duplicate National Weather Service Alerts

Table of Contents

1	Results in Brief	4

1.1	Executive Summary	4

2	Introduction	6

2.1	CSRIC VII Working Group 1 Team Members	7

3	Objective, Scope, and Methodology	8

3.1	Objective and Scope	8

3.2	Methodology	8

3.2.1	Emergency Alert System	9

3.2.2	Integrated Public Alert and Warning System	10

3.2.3	NOAA National Weather Service Dissemination of Weather Messages and Non-Weather Messages for broadcast over EAS	11

4	Definitions and Acronyms	12

4.1	Definitions	12

4.2	Acronyms	13

5	Identified Causes of Duplicate NWS Alerts	14

5.1	Location List: Relay-based EAS (Overlapping NWR Coverage)	14

5.2	Location List: CAP-based EAS	15

5.3	Timestamp	15

5.4	Different Originator IDs	15

5.5	Other Source Monitored for Alerts	15

5.6	Multiple CAP Messages Required for a Single Event	15

6	Potential Improvements and Solutions	16

6.1	Additional Data Tag	16

6.1.1	Implementation Decisions	18

6.1.1.1	Implementation Option 1: Modifying the existing EAS Header	18

6.1.1.2	Implementation Option 2: Adding Ancillary Data Without Modifying the EAS Header	18

6.2	Unblock NWS Alerts for distribution on the CAP EAS Channel for only the limited set of Geographic Areas where NWR is not available	20

6.3	Unblock NWS Alerts for distribution on the CAP EAS Channel for Single Geocode Alerts (CAP EAS Alerts with only one FIPS code)	21

6.4	Unblock NWS alerts for distribution on the CAP EAS Channel and Remove/eliminate NWR as a source for EAS	22

6.5	Unblock NWS alerts for distribution on the CAP EAS Channel: Establish CAP as the Primary NWS EAS source with NWR backup	23

6.6	NWS NOAA Weather Radio Sites provide Complete and Consistent Ordering of SAME Location (Local Area Codes) in All Alert Broadcast Messages	24

7	Conclusions	25

8	Recommendations	27

Annex A: CAP Audio Considerations	29

Annex B: FIPS Code Usage Data	31

Annex C: NWR Coverage Gaps	33

Annex D: Background and Illustrations	34

D.1 CAP/EAS Design	34

D.2 EAS Message Flow	35

Table of Figures

Figure 1 Emergency Alert System	9

Figure 2 IPAWS Architecture	11

Figure 3 – NOAA NWS Emergency Messages Delivery Paths for Broadcast over EAS	12

Figure 4 – NOAA NWS Non-Weather Emergency Messages over NOAA Weather Radio	13

Figure 5 – Depiction of NWS product transmission over NOAA Weather Radio where two separate NWR Transmitters include the same county	16

Table of Tables

Table 1 - Working Group Structure	6

Table 2 - List of Working Group Members	7

# Results in Brief

## Executive Summary

Under certain conditions, the public receives duplicate National Weather Service (NWS) alerts issued over the Emergency Alert System (EAS).  Duplicate alerts occur due to technical differences in the processes for generating, distributing and broadcasting of alerts between the NWS systems and EAS.  EAS devices are required to reject “duplicate” alerts when operating in automated mode, which is the typical mode of operation.  Some NWS alerts generated for a single weather event, however, can be received by broadcast stations and other entities that transmit EAS alerts to the public from multiple sources with variations (primarily with respect to the geographic areas to which the alert applies) that appear to EAS devices as discrete alerts rather than duplicate alerts covering the same weather event.

This report of CSRIC VII addresses the following:

The Federal Communications Commission (FCC) tasks CSRIC VII to recommend the overall best solution(s) to resolve the duplicate NWS alert issue.  CSRIC VII should comprehensively consider all aspects of the duplicate NWS alert issue, taking into consideration all relevant stakeholders’ concerns (including alert originators, EAS participants, NWS, Federal Emergency Management Agency (FEMA), as well as EAS equipment manufacturers and the public, the recipients of such alerts), and recommend the solution(s) that is the most effective, balancing the costs and benefits, for the majority of stakeholders.  As part of this process, CSRIC VII should involve all stakeholders in the alert initiation and transmission process in order to research and assess possible solutions, including the method proposed by CSRIC III.

For the purposes of this report, a duplicate alert is defined as the inability of EAS encoder/decoder equipment to disambiguate between two (or more) received variations of the same alert.  These variations occur as the alert traverses the National Oceanic and Atmospheric Administration (NOAA) Weather Radio All Hazards (NWR) system using Specific Area Message Encoding (SAME) and is modified into two or more EAS versions of the original alert.  Any change detected by the current method of a byte by byte comparison in any of the five EAS header fields will result in treating each variation of the alert as a new alert.  The most common variation occurs when localized encoding changes the length or order of the list of Location Codes, causing separate received instances of the alert to be determined, during the comparison process, to be distinct alerts.  Section 5 of the report provides details of the known causes of these variations.

Six proposals for changes that would improve the current field situation and, in some cases, mitigate the inability to detect when received alerts are actually variations of the same alert, have been analyzed.  Section 6 provides a description of each identified method, followed by a list of steps required to achieve the goal(s) described and identified Pros and Cons to be considered.  Where applicable and known, the need for additional quantifying information, testing, and criteria for making subsequent decisions are included.

The proposals identified lend themselves to two categories, as follows:

- Improvements with short-term (2-3 years) implementation, and

- Full, or near full, solutions requiring long-term (5+ years) implementation.

Following review and analysis of the findings of prior CSRICs, new proposals for improvements, and current field knowledge, CSRIC VII recommends that the FCC and EAS Industry pursue dual parallel paths—one near-term modification to increase access to Common Alerting Protocol (CAP) EAS alerts with little or no negative impact (e.g., audio, performance, capacity) to the EAS system, and one long-term solution with the potential to allow EAS devices which can be upgraded to have the ability to determine when they have received variations of the same alert, while limiting, to the extent possible, impacts to the current performance of EAS devices either unable or not planned to be upgraded.

CSRIC VII recommends the following two-pronged approach:

Recommendation #1: (Long-term solution) The FCC should consider adopting EAS rules supporting the solution described in Section 6.1 (Additional Data Tag), which adds an additional data tag to uniquely identify each alert.  This solution directly supports the ability to detect separate instances of the same alert, both the NWR-sourced and CAP-sourced, removing the need to block NWS weather alerts from proceeding through the CAP EAS Channel. While 6.1 is considered to be the preferred long-term solution because it offers the most complete solution for the identified issues if all EAS stakeholders comply, a full analysis of the solution described in section 6.6 (Complete and Consistent Ordering of SAME Location Codes) should be considered due to the limited impact to most stakeholders.

Recommendation #2: (Near-term improvement to EAS alert access) The FCC should consider adopting, in parallel with Recommendation #1, interim guidelines that will facilitate the passage of at least some NWS CAP messages via Integrated Public Alert and Warning System (IPAWS) to broadcasters without causing duplicate EAS broadcast.  The solution described in section 6.3 (Unblock the EAS Channel for Single Geocode Alerts) is the preferred near-term improvement with minimal impacts to EAS stakeholders.  The solution described in section 6.5 (Establish CAP as the Primary NWS EAS source with NWR backup) may be considered as an interim approach; however, more significant EAS stakeholder impacts exist, and this solution has a chance of generating CAP-sourced EAS duplicates for alerts that must be broken down due to limits defined by the protocols (e.g., greater than 31 FIPS codes or WEA 10/100 limits) for some types of wide area NWS events.

Final decisions on implementation details require further analysis by affected stakeholders.  Where applicable and known, the need for additional quantifying information, testing, and criteria for making subsequent decisions are included.  All recommendations for improvements and solutions will require close coordination (testing, hardware/software changes) throughout the entire dissemination value chain and any implementation of these recommendations will follow appropriate notification timelines.

# Introduction

The FCC directs CSRIC VII, via Working Group 1 (WG1), to recommend the overall best solution(s) to resolve the localization duplication issues that occur at the interface between the NWS NOAA Weather Radio broadcasts and transmissions over EAS distribution systems.

CSRIC VII Structure

| Communications Security, Reliability, and Interoperability Council (CSRIC) VII |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CSRIC VII Working Groups |  |  |  |  |  |
| Working Group 1: Alert Originator Standard Operating Procedures & Duplicate NWS Alert | Working Group 2: Managing Security Risk in the Transition to 5G | Working Group 3: Managing Security Risk in Emerging 5G Implementations | Working Group 4: 911 Security Vulnerabilities during the IP Transition | Working Group 5: Improving Broadcast Resiliency | Working Group 6: SIP Security Vulnerabilities |
| Chair: Craig Fugate, APTS/ CoChairs: Michelle Mainelli, NWS, Terri Brooks, T-Mobile | Chair: Kathy Whitbeck, Nsight | Chair: Farrokh Khatibi, Qualcomm | Chair: Mary Boyd, West Safety Services | Chair: Pat Roberts, Florida Association of Broadcasters | Chair: Danny McPherson, Verisign |
| FCC Liaison: James Wiley/ David Munson | FCC Liaison: Kurian Jacob | FCC Liaison: Steven Carpenter | FCC Liaison: Rasoul Safavian | FCC Liaison: Robert “Beau” Finley | FCC Liaison: Ahmed Lahjouji |

Table 1 - Working Group Structure

## CSRIC VII Working Group 1 Team Members

| Name | Representing |
| --- | --- |
| Michelle Mainelli-McInerney (Co-Chair) | National Weather Service |
| Terri L. Brooks (Co-Chair & Report Editor) | T-Mobile USA |
| Mark Annas | City of Riverside Fire Department – OEM |
| Joe Berry | California Broadcasters Association |
| Sulayman Brown | Fairfax County, VA |
| Wade Buckner | International Association of Fire Chiefs |
| Dana M. Carey | County of Yolo, CA |
| Edward Czarnecki | Digital Alert Systems, Inc. |
| Brian K. Daly | AT&T |
| John Davis | T-Mobile USA (Alternate) |
| Ashruf El-Dinary | Xperi Corporation |
| John Dooley | Minnesota Department of Public Safety (Alternate) |
| Clay Freinwald | Washington State SECC |
| Craig Fugate | America’s Public Television Stations |
| Mike Gerber | National Weather Service (Alternate) |
| Matthew Gerst | CTIA-The Wireless Association® |
| Robert Gessner | ACA Connects |
| Dana Golub | Public Broadcasting Service |
| Mark Hess | Comcast |
| Brian Hurley | America’s Communications Association |
| Antwane Johnson | FEMA |
| Chandra Kotaru | AWARN Alliance |
| Jeff Littlejohn | iHeartMedia Inc. |
| Alex McHaddad | Blue Mountain Translator District, OR |
| Peter Musgrove | AT&T (Alternate) |
| Michael Nix | Emergency Communications Authority |
| Jerry Parkins | Comcast (Alternate) |
| Donna Platt | NC Division of Services for the Deaf and Hard of Hearing |
| Harold Price | Sage Alerting Systems Inc. |
| Krisztina Pusok | American Consumer Institute |
| Pat Roberts | Florida Association of Broadcasters |
| Tim Romero | Sonoma County, CA |
| Craig Saari | Charter |
| Francisco Sanchez, Jr. | Harris County, TX |
| Timothy Schott | National Weather Service (Alternate) |
| Mark Schutte | Cox |
| Leslie Sticht | Minnesota Department of Public Safety |
| Jordan Villwock | Laguna Beach Police Department |
| John Williamson | Nez Perce Tribal Police Department |
| Jeff Wittek | Motorola Solutions, Inc. |
| FCC Liaison, David Munson |  |

Table 2 - List of Working Group Members

# Objective, Scope, and Methodology

## Objective and Scope

Alert systems in the U.S. serve a critical function, disseminating important and often life-saving information.  These alerts are targeted for specific areas or regions determined by the alert originators to be the area within which the public needs to receive the information being conveyed by the alert.

Maintaining the public’s faith and attention to this system is important if the alert system is to perform its intended function.  When the public receives a number of duplicate alerts, they may become ambivalent toward the alerts they receive.  CSRIC III referred to this in their report, as “audience fatigue.”  Safeguards to prevent duplicate alerts are in place, however, scenarios still occur which may result in the dissemination of duplicate alerts to the public.

CSRIC VII has been asked to involve all stakeholders in the alert initiation and transmission process in order to research and assess possible solutions, including the method proposed by CSRIC III, with the goal of recommending the solution that best balances costs and benefits.

## Methodology

This report sets forth common alert recommendations, as well as some recommendations, which address functionality applicable only to the EAS as it relates to delivery of weather-based emergency messages that originate from NOAA National Weather Service.

The next three sections provide an introduction to EAS, FEMA IPAWS, and NWS Dissemination Channels to activate EAS.

### Emergency Alert System

Figure 1 Emergency Alert System

The FCC, in conjunction with the FEMA and the NWS, implements the EAS at the federal level.  The EAS is the nation’s public warning system requiring broadcasters, cable television systems, wireless cable systems, satellite digital audio radio service (SDARS) providers, and direct broadcast satellite (DBS) providers to provide communications capability for the President to address the American public during a national emergency.   FEMA is responsible for a national-level activation of the EAS, tests, and exercises.

The Emergency Alert System was created in 1997, as a successor to the Emergency Broadcast System of the 1960s through 1990s, as a mechanism to pass along a Presidential Alert in the case of a national emergency. Over time, additional alerts have been added as optional alerts for more localized emergencies. These alerts were intended to be broadcast by TV and radio to the listening area.

Primary Entry Points (PEPs) are the primary source of the initial broadcast for a Presidential Alert.  They are the initial receiving entity of the Presidential Alert forwarded by FEMA.  Further along the communications chain, State Primaries and Local Primaries (LP-1, LP-2, etc…) may broadcast directly to the public, but are also monitored by stations not designated as PEPs, which will broadcast to the public.

The addition of video distribution by Multi-Channel Video Providers (MCVPs), including satellite and cable providers, allows the alerts to be propagated through the local broadcasters. In addition, the MCVP is required to carry the alert on other channels (programmed services) broadcast on the system. This requirement has evolved to cover Video on Demand and other video content. In a typical MCVP system, the alert will be distributed to every subscriber on the system. Recent updates require broadcasters and MCVP systems to poll IPAWS for alerts as well as listening to two State or Local Primary broadcasters.

Federal regulations require broadcasters to relay only one EAS alert message, the Presidential Emergency Alert Notification (EAN) as well as two test messages, the National Periodic Test (NPT) and the Required Monthly test (RMT) and originate a Required Weekly Test (RWT). It is voluntary for broadcasters to relay any of the other Event Codes, and this includes all of the weather messages NWS is authorized to relay to the EAS. The guidance on which voluntary messages are relayed to the EAS is generally detailed in State EAS Plans.

### Integrated Public Alert and Warning System

The figure below illustrates the fully integrated system of alerts supported by IPAWS.

Figure 2 IPAWS Architecture

“IPAWS provides public safety officials with an effective way to alert and warn the public about serious emergencies using the Emergency Alert System (EAS), Wireless Emergency Alerts (WEA), the National Oceanic and Atmospheric Administration (NOAA) Weather Radio, and other public alerting systems from a single interface. Federal, state, local, tribal and territorial alerting authorities can use IPAWS and integrate local systems that use Common Alerting Protocol (CAP) standards with the IPAWS infrastructure. View a list of IPAWS Organizations with Public Alerting Authority Completed in each state.”  [Source: www.fema.gov, updated 4/29/2020].

### NOAA National Weather Service Dissemination of Weather Messages and Non-Weather Messages for broadcast over EAS

The figure below provides a high-level overview of the dissemination path for NWS weather emergency messages for activation over EAS.

Figure 3 – NOAA NWS Emergency Messages Delivery Paths for Broadcast over EAS

For weather emergency messages, the FEMA IPAWS EAS Channel is blocked per NWS request due to the potential of further duplicate alerting.  The NWS activates the EAS most frequently for imminent and dangerous weather conditions.  The NWS uses NWR as its primary means to activate EAS with Specific Area Message Encoding (SAME) codes. With the exception of national-level activation of the EAS, it is voluntary for EAS participants, such as radio and television stations, to further relay NWS-generated messages.  The NWS also provides weather emergency messages via the NOAA Weather Wire Service and the Emergency Managers Weather Information Network (EMWIN) in Universal Graphic Code (UGC) format.

In some instances, the NWS is asked to amplify non-weather emergency messages on behalf of authorized alerting officials over NOAA Weather Radio.  Figure 4 below provides a high-level overview of the dissemination path for non-weather emergency messages when requested for broadcast over NWR.

Figure 4 – NOAA NWS Non-Weather Emergency Messages over NOAA Weather Radio

# Definitions and Acronyms

## Definitions

Alert Originator (AO): This is an entity authorized to create and send emergency messages.

Emergency Alert System (EAS): The Emergency Alert System was created as a successor to the Emergency Broadcast System as a mechanism to pass along a Presidential Alert and other optional types of alerts in the case of a national emergency.

Integrated Public Alert and Warning System (IPAWS): IPAWS provides public safety officials with an effective way to alert and warn the public about serious emergencies.

National Weather Service (NWS):  NWS provides weather, water, and climate forecasts and warnings for the United States, its territories, adjacent waters and ocean areas, for the protection of life and property and the enhancement of the national economy.

NOAA Weather Radio All Hazards (NWR): NWR is a nationwide network of radio stations broadcasting continuous weather information directly from the nearest National Weather Service office.  NWR may be used as an all-hazards alert source by EAS Participants.

Primary Entry Point (PEP): Primary Entry Points (PEPs) are the primary source of the initial broadcast for a Presidential Alert.

Wireless Emergency Alert (WEA): The Wireless Emergency Alert system allows customers who own compatible mobile devices to receive geographically targeted alert information to warn them about imminent threats to safety or other critical situations.

## Acronyms

| AO | Alert Originator |
| --- | --- |
| CAP | Common Alerting Protocol |
| DBS | Direct Broadcast Satellite |
| EAN | Emergency Alert Notification |
| ECIG | EAS-CAP Industry Group |
| EAS | Emergency Alert System |
| EMWIN | Emergency Managers Weather Information Network |
| FEMA | Federal Emergency Management Agency |
| IPAWS | Integrated Public Alert and Warning System |
| LP | Local Primary |
| MCVP | Multi-Channel Video Provider |
| NOAA | National Oceanic and Atmosphere Administration |
| NWR | NOAA Weather Radio All Hazards |
| NWS | National Weather Service |
| PEP | Primary Entry Point |
| SAME | Specific Area Message Encoding |
| SDARS | Satellite Digital Audio Radio Service |
| SECC | State Emergency Communications Committee |
| SP | State Primary |
| SR | State Relay |
| SRN | State Relay Network |
| WEA | Wireless Emergency Alert |
| WFO | Weather Forecast Office |
| WMO | World Meteorological Organization |

# Identified Causes of Duplicate NWS Alerts

This section of the report describes the identified scenarios that cause duplicate NWS alerts to be received by the public.  These scenarios were also noted, studied, and presented by CSRIC III in Part 4 of their Report, with the exception of the item concerning multiple CAP messages for a single event. ,

In EAS, a byte-by-byte comparison is performed to detect duplicates.  If any information in the EAS message is different than the previously received and stored messages (with the exception of the transmitting or retransmitting identifier), it will not be deemed a duplicate.

The following elements are compared as part of the duplication detection:

- Originator code

- Event code

- List of location codes

- Start time

- Duration

## Location List: Relay-based EAS (Overlapping NWR Coverage)

In order to ensure that the entire public receives the alert, NOAA Local Transmitters may overlap in their broadcast areas, however, the full description of their broadcast areas (e.g., list of counties) will differ.  In the overlap area, two transmissions carrying the same event information will not be detected as duplicates because the accompanying location information is not an exact match.  A graphical depiction of this scenario is shown in Figure 5.

Figure 5 – Depiction of NWS product transmission over NOAA Weather Radio where two separate NWR Transmitters include the same county

This type of duplication is limited due to natural “geographic barriers” in many areas, and because receiving stations rarely monitor two NWR Transmitters.

## Location List: CAP-based EAS

NWR is designed to broadcast relevant information to a listening audience for and in specific geographic areas, however, CAP messages are sent to every EAS participant.  Every station can “hear” both the CAP alert and the version transmitted by their local NWR transmitter.  In many cases the location codes in the NWR broadcast will only contain a subset of the locations covered in the IPAWS CAP message and the order of the location codes may be different. If the NWR transmitter sends a subset of the full location list in the CAP message to EAS, the CAP and NWR broadcasts will be perceived by the receiving equipment as distinct alerts even though they are duplicates.  Both versions could be aired by broadcast stations and cable head-ends.

Out of a total of 214,991 alerts that could have been sent to CAP for EAS distribution in 2020, 146,292 (68%) had more than one location, and could be subject to localization caused duplication.

## Timestamp

A single source of information may result in the generation of multiple CAP messages with different sent times.  This concern has been resolved by current practice.

## Different Originator IDs

Two different originators may send the same event information with different time stamps and different location lists. This concern has been resolved by current practice.

## Other Source Monitored for Alerts

Broadcasters for EAS activation may monitor sources other than NWR and IPAWS.  For example, several state relay networks monitor and/or combine alerts in various formats for distribution. The risk of duplicate EAS alerts is increased when alerts in multiple formats are distributed using other methods, especially when those methods involve localization by including only a subset of the list of Location Codes in the alert.

## Multiple CAP Messages Required for a Single Event

While a single CAP message will ideally account for an entire alert area, it is not always possible  to convert this to EAS due to the 31 location code limitation in EAS and SAME Multiple CAP messages may be needed to account for a single event spanning a large area or areas when the location of the threat can be conveyed with a high degree of accuracy (e.g. storm surge and tsunami). In this scenario, broadcaster equipment is unable to detect that the multiple CAP messages received are merely a subset of the larger alert area and a duplicate EAS alerting issue arises. Note: The CAP EAS Channel is currently closed for weather alerts initiated by NWS.

There are two identified constraints which may result in the need to use multiple CAP messages rather than one:

- An alert area spanning greater than 31 FIPS codes, and

- The WEA 10/100 limits.

Data from NWS alerts in 2020 show that only 0.03% of all warnings are of the type that would be sent to EAS and include more than 31 location codes, 20 messages in all.  A larger number of watches and statements fall in this category, 1%, or 1,691 out of a total of 154,410 alerts of those types.

No specific data has been analyzed to determine the number of NWS alerts that may need to be broken down into multiple alerts due to the WEA 10/100 limits.  This rule limits the number of shapes (polygons and circles) for one alert message to 10, and the total number of coordinates to define the included shapes to 100. If an alert message needs to convey information which would pass these limits, it must be broken into multiple alerts.

# Potential Improvements and Solutions

This section contains descriptions and an analysis for each potential action or set of actions identified as offering a solution or some form of improvement.

The solutions in this section involve the introduction of NWS weather alerts coming through the CAP EAS Channel.  This raises the aspect of taking a closer look at the guidance and implementation needed to move this change forward successfully.  CAP EAS provides improvement in Audio clarity with local text-to-speech to reduce distortion verses daisy chain pass-through audio degradation, although certain pronunciation, abbreviation, ellipses, and other formatting issues will need to be addressed (See Annex A).

## Additional Data Tag

CSRIC VII, as have other CSRICs before it, has identified the primary cause of the undetectable duplicate issue to be the localization of alerts at the interface between NWR systems and EAS systems.  This causes two or more versions of the original message to be transiting EAS concurrently. There is insufficient information to allow disambiguation of the different versions in the current design of the EAS system.

In other words, EAS lacks a single identifying piece of information which can be compared to determine whether the alert has been previously received.

The most complete solution to the problem is to define the necessary additional information, referred to here as a “data tag”, and to specify a method of passing the data tag to and through EAS, providing originating systems, systems that bridge between NWS and EAS, EAS relays, and EAS receivers with an element that is common to all versions of an alert so that EAS receivers can disambiguate.

The main challenge of this solution is allowing the new data tag to be sent without disrupting legacy devices, allowing an orderly transition path from old to new systems, and to allow legacy and new systems to run in parallel for some time to come.

This section explores key decision points, as well as two implementation options for this particular solution.  The Steps Required for Implementation shown below apply to either option.

Steps Required for Implementation

| FCC | Modification to rules to define changes to EAS protocol. |
| --- | --- |
| Originators (NWS and others where localization might occur) | Will need to provide/generate the data tag information. |
| EAS participants | Any EAS participant that will transmit NWR alerts should update software, or accept the potential of duplicates. Any EAS Participants that relays alerts to other EAS Participants - SP, SR, SRN, or LP, must upgrade to permit downstream stations to detect duplicates. |
| Origination Software providers | Provide/generate data tag information, both in CAP and in direct legacy EAS origination. |
| EAS software/hardware providers | Receive data tag from CAP and EAS, use it to disambiguate, and insert it on (re)transmission. |
| Backward Compatibility Testing | Confirm compatibility with legacy equipment (existing Public Alert™ and NWR receivers) to ensure that legacy functions are not impaired. |
| FEMA | Define or concur with data tag in IPAWS CAP messages if such is required. Remove block of NWS messages to EAS. |
| ECIG or another similar group | Standardization is required to define construction and use of data tag for duplication detection. [Part 11 or by reference to ECIG or similar standards body.] |
| NWS | SAME specification changes and software/hardware changes to conform to the new design. Could include generation of CAP messages as well as transmitted the data tag on NWR |
| SECC | State plan must note that relay points must update to handle data tags for effective use of this solution to detect duplicates. |

### Implementation Decisions

There are two key decision points:

- The content and source of the data tag: While one possible choice is to use the CAP message ID, not all messages originate in CAP format, for example, NWR messages currently originate in WMO format.  Study will be required.  If use of a CAP ID is not feasible, data generated specifically to create the data tag could be added to a CAP message when it is imported from the non-CAP source.  This decision point applies regardless of the implementation option chosen.

- The method of including the data tag in an EAS message: There are two basic implementation options to add a data tag. The data tag may be added to an EAS message through the addition of data frames added before or within the existing EAS message.  These implementation options are outlined and examined below in Implementation option 2.

#### Implementation Option 1: Modifying the existing EAS Header

Though discussed in the CSRIC III report in 2013, modifying the existing EAS header (i.e., a change in the EAS Protocol) entails a substantial change to the existing header string through the addition of new bits.  Current thoughts for the length of the data tag eight years later have caused  CSRIC VII to reexamine that option.

That option is viewed as not being an acceptable implementation option for the Additional Data Tag solution.  It is the most disruptive option because of its adverse impact on systems and equipment (including NOAA Weather Radios) that could cease to process EAS messages correctly with such a change.  Though CSRIC VI included this implementation option in their discussions in their Report on Comprehensive Re-imagining of Emergency Alerting– Security Aspects of Emergency Alerting System,” we now view this as too disruptive of a change to be considered.

#### Implementation Option 2: Adding Ancillary Data Without Modifying the EAS Header

Within this approach, there are multiple techniques that could be used to insert additional data outside of the EAS header.  These techniques were discussed by CSRIC VII, including techniques which have already implemented in the field to differing extents.  Two approaches that received particular discussion in CSRIC VII include:

- Adding data after EAS header and attention signal: <EAS header> <attention tone> <data frame> <audio> <EOM>

- Adding data before the EAS header: <data frame> <EAS header> <attention tone> <audio> <EOM>

Techniques along these lines were previously recommended for evaluation by CSRIC VI in their Report on Comprehensive Re-imagining of Emergency Alerting.

CSRIC VII recommends that the FCC and industry undertake a process of evaluation of the alternative techniques of inserting data into an EAS message, as described above, including field testing and independent evaluation of any proposed data insertion techniques by the FCC and/or its Federal partners.  Each technique may have different technical strengths and challenges, including the possible inability of some equipment to adapt and use the approach.

| PROS | CONS |
| --- | --- |
| This solution offers a potential full solution to the task assigned to this WG. Unmodified systems could continue to originate valid emergency alerts without ancillary data outside the EAS header. This implementation option would not impede the operation of EAS encoder/decoders at EAS Participant sites that – for whatever reason – have not updated their systems, or whose manufacturers do not offer an update for this technique. This implementation option would not force alert originators at the Federal, state, local, territorial and tribal levels to modify their EAS origination systems. They may update their systems at their discretion. Alert monitoring solutions (e.g. NOAA Weather Radios and unsupported EAS devices) that are not configured to process data outside the EAS header would simply ignore that data (i.e. the alert would not otherwise be impeded with the presence of this added data). If an extensible data tag capability is added to EAS, several other EAS additions considered by the FCC, e.g. CSRIC VI could use the same facility, adding additional benefits to balance the cost of adding ancillary data to EAS. Note: The above Pro statements concerning backwards compatibility are set forth based on expert knowledge of the system, but the Steps for Implementation include testing for verification. | EAS Participants that do not update their software/firmware will see no improvement. Either approach would extend the overall length of an EAS message from a fraction of a second to several seconds. EAS Participants designated in their State Plan as State or Local primary who choose not to update will propagate duplication issues to downstream participants. Cost/resource impacts of integration, conformance certification and end-to-end system capability verification from many EAS stakeholders |

## Unblock NWS Alerts for distribution on the CAP EAS Channel for only the limited set of Geographic Areas where NWR is not available

NWS removes the CAP EAS block channel for limited geographic areas that do not have NWR SAME broadcasting available. EAS participants in other areas would continue to rely on NWR SAME would not receive alerts on the IPAWS EAS Channel.

Steps Required for Implementation

| NWS, FEMA/IPAWS, SECC and EAS Participants | Establish a process for identifying geographic areas (FIPS codes) to include on the “allowed list” for CAP EAS channel use such that the NWS block channel settings satisfy the needs of the EAS community. *Note: Third party EAS sources pose a risk based on their translation to the EAS protocol. |
| --- | --- |
| NWS | Develop software to provide configurations that support targeted unblocking of the CAP EAS channel. |

| PROS | CONS |
| --- | --- |
| Limited implementation impacts. EAS reaches geographic areas not currently receiving EAS alerts. | Limited geographic areas would benefit (See Annex C). Requires significant time for well-disciplined collaboration and coordination between NWS, FEMA/IPAWS, SECC and EAS user community to establish and maintain the allowed list of Geographic Areas. NWS development time and deployment costs. Solution not suitable for alerts covering multiple FIPS codes if one or more are not on the allowed list, resulting in no improvement for the percentage of alerts in this category. Does not address duplicate detection for certain originated alert events (e.g., with large geographic areas such as Storm Surge/Tsunami) that may need to be broken out into multiple alerts to meet either the WEA 10/100 rule or to not exceed a maximum of 31 FIPS for compatibility to existing receivers and equipment. |

## Unblock NWS Alerts for distribution on the CAP EAS Channel for Single Geocode Alerts (CAP EAS Alerts with only one FIPS code)

NWS and FEMA/IPAWS will remove the block for NWS alerts transiting the EAS Channel for alerts containing only one FIPS code to the IPAWS EAS channel as single-location alerts are not subject to localization.  NWS and FEMA/IPAWS continue to block alerts containing multiple FIPS codes from distribution on the IPAWS EAS channel.

Steps Required for Implementation

| NWS and FEMA/IPAWS | NWS and FEMA IPAWS software/configuration modification to detect and allow alerts with a single FIPS code. |
| --- | --- |
| EAS Participants | Notification will need to be given to EAS Participants. They will need to review settings to ensure their ability to receive and accept/reject these CAP alerts. |
| EAS Manufacturers | Notification will need to be given to EAS Manufacturers, allowing time for the addition of options in some cases needed by users to control CAP vs EAS received alerts. |
| SECC | Review and update state plans as needed. |
| FCC, NWS and FEMA | Outreach program with end goal of educating EAS Participants and the general public in order to set expectations and understanding. |

| PROS | CONS |
| --- | --- |
| NWS alerts would begin reaching some recipients currently not receiving alerts, with no increased duplication for any recipients. Data analysis indicates this method will allow approximately 40% of NWS Warn class alerts to pass through IPAWS. (See Annex B) | Does not directly address the current inability to identify multiple instances of the same alert. Viewers/listeners of broadcasters who rely solely on the IPAWS EAS channel will not receive EAS notification for alerts covering multiple SAME location codes. Receipt of some alerts may create the expectation that they will now receive all alerts. |

## Unblock NWS alerts for distribution on the CAP EAS Channel and Remove/eliminate NWR as a source for EAS

NWS and FEMA/IPAWS will remove the block for NWS alerts transiting the EAS Channel for.  NWR is not permitted as an alert source for EAS participants.

Steps Required for Implementation

| NWS and FEMA/IPAWS | NWS and FEMA/IPAWS software/configuration modification to remove block on CAP EAS Channel |
| --- | --- |
| EAS Participants | Notification will need to be given to EAS Participants. They will need to review settings to ensure their ability to receive and accept or reject these CAP alerts. Software/hardware/reconfiguration changes to stop receiving weather alerts from NWR source. |

| PROS | CONS |
| --- | --- |
| NWS alerts available through the IPAWS EAS channel and third-party distribution and do not result in duplicate detection issues at the broadcaster when no NWR SAME localized sources are used. | Loss of resilience in EAS monitoring, with some EAS Participants (those without an alternate source other than NWR) left with only the CAP source. Does not address duplicate detection for certain originated alert events (e.g., with large geographic areas such as Storm Surge/Tsunami) that may need to be broken out into multiple alerts to meet either the WEA 10/100 rule or to not exceed a maximum of 31 FIPS for compatibility to existing receivers and equipment. |

## Unblock NWS alerts for distribution on the CAP EAS Channel: Establish CAP as the Primary NWS EAS source with NWR backup

NWS and FEMA/IPAWS will remove the block for NWS CAP alerts to the EAS Channel.  EAS participants will only use NWR as an alert source if their EAS device is unable to check for a matching CAP EAS alert when an NWR SAME alert arrives because CAP is not available (e.g., service down).

Steps Required for Implementation

| NWS and FEMA/IPAWS | NWS and FEMA/IPAWS software/configuration modification to remove block on CAP EAS Channel. |
| --- | --- |
| Alert Origination System Providers, EAS Manufacturers, EAS Participants | Develop practices (software changes or other) to determine whether CAP is down and whether to accept a received EAS-sourced alert. |

| PROS | CONS |
| --- | --- |
| Use of CAP EAS as a primary source mitigates the duplication detection difficulties for any alerts received while CAP is available. | EAS device software change to accommodate the CAP available/not available decision and subsequent actions. Does not address duplicate detection for certain originated alert events (e.g., with large geographic areas such as Storm Surge/Tsunami) that may need to be broken out into multiple alerts to meet either the WEA 10/100 rule or to not exceed a maximum of 31 FIPS for compatibility to existing receivers and equipment. |

## NWS NOAA Weather Radio Sites provide Complete and Consistent Ordering of SAME Location (Local Area Codes) in All Alert Broadcast Messages

NWS implements Alert message on all NWR Audio broadcasts with the full and original SAME code and in the original order sequence thereby eliminating any localization (i.e., variations) of the message header (number of FIPS codes and order sequence) content.  This change will ensure all SAME alert messages from NWR transmitters would have identical header content, making them easily detectable as duplicates by EAS devices.  With this modification, the header content will also match that of the messages received by IPAWS for further distribution on the CAP EAS Channel.

Steps Required for Implementation

| NWS | NWS software development would be required to provide changes in the selection and ordering of local area FIPS codes included in NWR SAME messages (local Area Codes) |
| --- | --- |

| PROS | CONS |
| --- | --- |
| Consistent ordering of local area codes in NWR SAME message across different transmitters provides a straightforward change to allow proper duplicate detection when alerts are received from multiple NWR transmitters. Having NWR broadcasts that have identical header content as EAS sources provides a straightforward change to allow proper duplicate detection when alerts are received from NWR and third party EAS sources. This change not necessitate any coding changes by EAS device manufacturers for improved broadcast capability. Allows enabling of weather alerts on the IPAWS-OPEN EAS channel as an EAS weather message alert source for EAS participants. | Changes to NWR transmitter broadcast content require time and costs/resources for development and deployment. Does not address duplicate detection for certain originated alert events (e.g., with large geographic areas such as Storm Surge/Tsunami) that may need to be broken out into multiple alerts to meet either the WEA 10/100 rule or to not exceed a maximum of 31 FIPS for compatibility to existing receivers and equipment. |

# Conclusions

The task being addressed by CSRIC VII is not a newcomer to the FCC CSRIC process.  The end-to-end alert issues which result in the inability to detect the reception of two variations of the same alert has been discussed by multiple iterations of CSRIC, in addition to other forums.  No short-term solution with the potential to eliminate or even significantly reduce the inability to detect these variations has been identified to date.

Following review and analysis of the findings of prior CSRICs, new proposals for improvements, and current field knowledge, CSRIC VII feels that a two-pronged approach (two parallel paths) should be followed, given:

- the wide range of estimated implementation periods for the potential solutions or improvements identified,

- the uncertainty of the exact level of impact to some stakeholders,

- the quantifying data still needed to determine the extent of the improvement truly effected by some proposed methods, as noted in the appropriate section per method,

- the impartial testing required to verify the current capabilities of, and the extent (risk) of the impact to, existing equipment (i.e., the ease or ability of that equipment to benefit from, or not be functionally impaired by, a given change or set of changes),

- the uncertainty of the weight given to each of the above-mentioned factors by the FCC.

The proposals identified lend themselves to two categories, as follows:

- Improvements with short-term (2-3 years) implementation, and

- Full, or near full, solutions requiring long-term (5+ years) implementation.

The conclusion of CSRIC VII is that it would be best to pursue, in parallel, both a short-term improvement and a solution that offers more significant improvement in the long-term.

# Recommendations

CSRIC VII recommends that the FCC and EAS Industry pursue dual paths—one near-term modification to increase access to CAP EAS alerts with little or no negative impact (e.g., audio, performance, capacity) to the EAS system, and one long-term solution with the potential to allow EAS devices which can be upgraded to have the ability to determine when they have received variations of the same alert, while limiting, to the extent possible, impacts to the current performance of EAS devices either unable or not planned to be upgraded.  The preferred goal is no negative impact to the current performance, not only for the sake of equipment that either cannot or may never be upgraded but allowing a reasonable conversion timeframe by those that will upgrade without interrupting their EAS handling in the meantime.

CSRIC VII recommends the following two-pronged approach:

Recommendation #1: (Long-term solution) The FCC should consider adopting EAS rules supporting the solution described in Section 6.1 (Additional Data Tag), which adds an additional data tag to uniquely identify each alert.  This solution directly supports the ability to detect separate instances of the same alert, both the NWR-sourced and CAP-sourced, removing the need to block NWS weather alerts from proceeding through the CAP EAS Channel. While 6.1 is considered to be the preferred long-term solution because it offers the most complete solution for the identified issues if all EAS stakeholders comply, a full analysis of the solution described in section 6.6 (Complete and Consistent Ordering of SAME Location Codes) should be considered due to the limited impact to most stakeholders.

Implementation of 6.1 will require hardware and/or related software changes throughout the entire EAS stakeholder communications chain.  Section 6.1 includes a detailed list of known required changes per stakeholder.  While Section 6.1 identifies the required tasks of the overall solution, an overview of two preferred implementation variations is also included.  Criteria, including testing to identify level of impact to current field equipment, are included to guide further steps toward decision-making at the implementation level.

Recommendation #2: (Near-term improvement to EAS alert access) The FCC should consider adopting, in parallel with Recommendation #1, interim guidelines that will facilitate the passage of at least some NWS CAP messages via IPAWS to broadcasters without causing duplicate EAS broadcast. The solution described in section 6.3 (Unblock the EAS Channel for Single Geocode Alerts) is the preferred near-term improvement with minimal impacts to EAS stakeholders.  The solution described in section 6.5 (Establish CAP as the Primary NWS EAS source with NWR backup) may be considered as an interim approach; however, more significant EAS stakeholder impacts exist, and this solution has a chance of generating CAP-sourced EAS duplicates for alerts that must be broken down due to limits defined by the protocols (e.g., greater than 31 FIPS codes or WEA 10/100 limits) for some types of wide area NWS events.

Final decisions on the exact improvements and/or solutions to fully pursue and the implementation details require further analysis by affected stakeholders.  Where applicable and known, the need for additional quantifying information, testing, and criteria for making subsequent decisions are included.  All recommendations for improvements and solutions will require close coordination (testing, hardware/software changes) throughout the entire dissemination value chain and any implementation of these recommendations will follow appropriate notification timelines.

If the Additional Data Tag (6.1) solution is pursued to completion, CSRIC VII recommends that the implementation chosen, if feasible, be extensible in order to provide for future modifications to the EAS system without the extensive deliberations and impacts that have been required to solve the current issues.

# Annex A: CAP Audio Considerations

Before the block on CAP NWS alerts for EAS is removed (both at NWS and at FEMA), the NWS needs to issue guidance on how to best handle conversion of CAP text to speech and video crawl.  Industry needs time to modify software to implement this guidance. The NWS uses formatting, such as ellipses and ampersands, tables, URLs and abbreviations in some of its alerts.  Intelligibility of the alert when converted to speech or a crawl could be improved by coordination with EAS manufactures.

The NWS text in CAP messages serves many users of weather alerts, not just EAS.  EAS, on the other hand, receives alerts from many different originators, and has a set of rules that limits the number of CAP elements used to derive its crawl and text to speech input. In many cases, CAP originators have evolved with EAS already in place.  NWS text and formatting preexists EAS and CAP by many years.  This has led to a problem with interfacing the two systems.

As an example to illustrate the challenges, here is a flood warning from February 16, 2021, with text below constructed per Part 11 rules.  The first sentence, containing FCC required elements, will vary from manufacturer to manufacturer.  The rest is verbatim from the description and instruction elements of the CAP message as defined for use for EAS:

The National Weather Service has issued a Flood Warning for Edgecombe, NC beginning at 10:11 am and ending at 10:15 pm.

...The Flood Warning continues for the following rivers in North Carolina...

Tar River At Tarboro affecting Edgecombe County.

...The Flood Warning remains in effect...

The Flood Warning continues for

the Tar River At Tarboro.

* Until further notice.

* At 9:15 AM EST Tuesday the stage was 21.3 feet.

* Flood stage is 19.0 feet.

* Minor flooding is occurring and moderate flooding is forecast.

* Forecast...The river is expected to rise to a crest of 26.3 feet

early Friday morning.

* Impact...At 20.0 feet, Lowland flooding begins south of Daniel

Street in east Tarboro.

* Impact...At 24.0 feet, Moderate flooding begins in Tarboro. River

Road is underwater and flooding begins on east Wilson and St David

Streets.

* Flood History...This crest compares to a previous crest of 25.6

feet on 02/12/2020.

&&

Fld   Observed           Forecasts (1 pm EST)

Location Stg   Stg   Day/Time     Wed   Thu   Fri   Sat

Tar River

Tarboro  19.0  21.3  Tue 9 am EST 24.8  26.1  26.3  25.9

&&

Turn around, don`t drown when encountering flooded roads. Most flood deaths occur in vehicles. Additional information is available at www.weather.gov/rah/rivers. The next statement will be issued this evening by 1015 PM EST.

Though some video EAS displays can present data in page format, a linear crawl a common display method.  ECIG specifies removal of repeated white space characters, and in general collapsing the data into a single line.  Multi-line tables with headers become garbled when presented this way, both visually and as input to text to speech.  Note also the ampersands, which have no special meaning to EAS, and will be spoken as “and” in many cases.

Web site URLS in text to speech are also often sometimes problematic.

Even video systems that render an alert in page format will generally have no cues on where to place page breaks, resulting in table headers being separated from the data.

As we move toward placing NWS CAP on the IPAWS EAS feed, the NWS and the EAS manufactures need to work together to present CAP NWS alerts (the single largest source of EAS alerts) effectively to the EAS audience.  Discussion on this is required to avoid audience and EAS Participant complaints about intelligibility, and possible tune-out or no carry.

# Annex B: FIPS Code Usage Data

Summary of FIPS Counts by Event Class

(NWS Alerts Sent to IPAWS in 2020)

| FIPS Codes | Warn | Watch | Stmt | Total |
| --- | --- | --- | --- | --- |
| 1 | 24099 | 5430 | 39170 | 68699 |
| 2-31 | 36462 | 11814 | 96305 | 144581 |
| 32+ | 20 | 649 | 1042 | 1711 |
| Total | 60581 | 17893 | 136517 | 214991 |

Percentage of FIPS Counts Within Event Class

(NWS Alerts Sent to IPAWS in 2020)

| FIPS Codes | Warn | Watch | Stmt |
| --- | --- | --- | --- |
| 1 | 39.78% | 30.35% | 28.69% |
| 2-31 | 60.19% | 66.03% | 70.54% |
| 32+ | 0.03% | 3.63% | 0.76% |
| Total | 100% | 100% | 100% |

Notes for above charts:

Data from OpenFEMA IPAWS archive

https://www.fema.gov/openfema-data-page/ipaws-archived-alerts-v1-0

Alerts counted are for EAS originator WXR with a valid Part 11 Event Code

Data includes CAP message of msgType Alert or Update

Total NWS Alerts by CAP msgType

(NWS Alerts Sent to IPAWS in 2020)

| Alert | 101853 |
| --- | --- |
| Update | 113138 |
| Cancel | 44985 |
| Total | 259976 |

Notes for above chart:

Data from OpenFEMA IPAWS archive

https://www.fema.gov/openfema-data-page/ipaws-archived-alerts-v1-0

Alerts counted are for EAS originator WXR with a valid Part 11 Event Code

Complete list of currently permitted EAS codes 47 CFR Part 11.31(c)

| ADR | Administrative Message | HWW | High Wind Warning |
| --- | --- | --- | --- |
| AVA | Avalanche Watch | LAE | Local Area Emergency |
| AVW | Avalanche Warning | LEW | Law Enforcement Warning |
| BLU | Blue Alert | NIC | National Information Center |
| BZW | Blizzard Warning | NMN | Network Message Notification |
| CAE | Child Abduction Emergency | NPT | National Periodic Test |
| CDW | Civil Danger Warning | NUW | Nuclear Power Plant Warning |
| CEM | Civil Emergency Message | RHW | Radiological Hazard Warning |
| CFA | Coastal Flood Watch | RMT | Required Monthly Test |
| CFW | Coastal Flood Warning | RWT | Required Weekly Test |
| DMO | Practice/Demo Warning | SMW | Special Marine Warning |
| DSW | Dust Storm Warning | SPS | Special Weather Statement |
| EAN | Emergency Action Notification | SPW | Shelter in Place Warning |
| EQW | Earthquake Warning | SSA | Storm Surge Watch |
| EVI | Evacuation Immediate | SSW | Storm Surge Warning |
| EWW | Extreme Wind Warning | SVA | Severe Thunderstorm Watch |
| FFA | Flash Flood Watch | SVR | Severe Thunderstorm Warning |
| FFS | Flash Flood Statement | SVS | Severe Weather Statement |
| FFW | Flash Flood Warning | TOA | Tornado Watch |
| FLA | Flood Watch | TOE | 911 Telephone Outage Emergency |
| FLS | Flood Statement | TOR | Tornado Warning |
| FLW | Flood Warning | TRA | Tropical Storm Watch |
| FRW | Fire Warning | TRW | Tropical Storm Warning |
| HLS | Hurricane Statement | TSA | Tsunami Watch |
| HMW | Hazardous Materials Warning | TSW | Tsunami Warning |
| HUA | Hurricane Watch | VOW | Volcano Warning |
| HUW | Hurricane Warning | WSA | Winter Storm Watch |
| HWA | High Wind Watch | WSW | Winter Storm Warning |

# Annex C: NWR Coverage Gaps

NWR transmitter broadcasts are not available for 36 counties (FIPS) in 11 states (does not including US Territories):

AK - 5
CA - 2
CO - 5
HI - 1
ID - 3
KY - 1
LA - 2
NM - 5
OR - 2
TX - 9
UT - 1

The counties without NWR coverage tend to be located in sparsely populated areas.

# Annex D: Background and Illustrations

## D.1 CAP/EAS Design

## D.2 EAS Message Flow

EAS devices identify duplicate alerts by comparing the EAS header code information – which specifies the alert event type, alert originator, affected geographic areas and other information – against alerts previously received.  If an alert contains identical EAS header code information when compared to a prior alert, the EAS device will reject that subsequent alert as a duplicate.

See CSRIC III Report on Alerting Issues Associated with CAP Migration, Parts 1-4 (2012-2013).

See CSRIC III Report on Alerting Issues Associated with CAP Migration, Parts 1-4 (March 2013).

CAP Handler will replace HazCollect (NOAA) and is scheduled for deployment in early 2021.

See CSRIC III Report on Alerting Issues Associated with CAP Migration, Part 4 (March 2013).

See Annex B.

See Annex B.

See CSRIC III Report on Alerting Issues Associated with CAP Migration, Part 4 (March 2013).

See CSRIC VI Report on Comprehensive Re-imagining of Emergency Alerting– Security Aspects of Emergency Alerting Systems (December 2018).

See CSRIC VI Report on Comprehensive Re-imagining of Emergency Alerting– Security Aspects of Emergency Alerting Systems (December 2018).